Model 485 wind speed sensor an instruction manual

JXBS-3001-FS

Ver1.0

# Chapter 1 product introduction

  - Product overview

Jxbs-3001-fs series wind speed sensor is small and light in appearance, easy  to carry and  assemble. The three cup design concept can effectively obtain  the  external  environment  information.  The  shell is made of high-quality polycarbonate material, and the  internal  smooth  bearing  system ensures the accuracy of information collection. It is widely used in greenhouse, environmental protection, weather  stations, ships, docks,  aquaculture  and other  environment  wind speed measurement.

### Main parameter

Parameter	Technical index

Wind speed measurement range

Wind speed measurement accuracy

0-30m/s

±1m/s

response time	less than5seconds

Baud rate	9600

Communicati on port

### RS485

Power supply	12V-24V DC

## consume power Operating temperature

<1W

### -30-80℃

Working humidity environment

0-100%RH（15-95%RH）

  - system framework diagram

Chapter 2 hardware connection

  - inspection before equipment installation

Please check the equipment list before installing the equipment:

Name	number

High	precision sensor

Wind	speed	485 line

12V	waterproof power supply

1 set

1 wire 1(optional)

USB to 485 device	1(optional)

Warranty	card	/	1 copy

certificate

    - wiring：

##### Line colou r

Note

Powe r suppl y Com

munic ations

Red	Positive power

Black	Negative power

Yellow	485A 485B

Green

Factory default to provide 0.6 meters long wire, customers can extend wire or sequential wiring as needed.

  - Installation mode

Using flange installation, threaded flange connection makes the lower pipe fittings of wind speed sensor firmly fixed on flange, four mounting holes are opened on the circumference of chassis, and bolts are used to fasten them tightly to the support. Ensure the accuracy of wind direction data, flange connection easy to use, can withstand greater pressure.

    - fixation

# The sensor size is shown

below:

Main Drawing

Dimensions

### Base dimensions

According to the size of the sensor, you  can  install  the sensor to the right position 。

Chapter 3 Communications Agreements

  - Basic communication parameters

Parameters	Content

Code	8-bit binary

Data bits	8-bit

parity bit	No

Stop position	1 bit

Error calibration

Error

calibration

baud rate	2400bps/4800bps/9600   bps	Available,

### factory default is 9600 bps

Code	8 Bit binary

  - Data frame format definition

Adopt the Modbus-RTU protocol as follows:

Initial structure >=4 bytes time Address =1 byte

Function =1 byte Data =N bytes

Error check =16-bit CRC code End structure >=4 bytes of time

Address code: the address of the transmitter, unique in the inquiry network (factory default 0 x01).

Function code: the host sends the instruction function prompt, this transmitter only uses the function code 0 x03( reads the memory data).

Data area: data area is a specific query area, note that 16 bits data high byte before CRC code: two-byte check code.

### Question frame

#### Register

Check	theCheck the high

#### Address Functi Functional length

low bit

bit

#### code	onal	code

code

1 bytes	1 bytes  2 bytes	2 bytes	1 bytes	1 bytes

Response frames

Function   Number     ofData area I	Second	Data N data area

Address al code code

valid bytes

Area

1bytes   1bytes   1bytes	2bytes	2bytes	2bytes

  - Register address

| Register address | PLC Configura tion Address | content | Operatio n |
| --- | --- | --- | --- |
| 0000H | 40001 | Wind speed (0.1m/s ) | Read |
| 0100H | 40101 | Device address(0-252) | only Read |

0101H	40102	baud	rate

(2400/4800/9600)

and write Read and write

  - Examples	of	communication	protocols	and explanations

    - read the wind speed value x01 device address 0 Question frame

| Addre ss code | Funct ional code | Starting address | Data length | Check the low bit | Check the high bit |
| --- | --- | --- | --- | --- | --- |
| 0x01 | 0x03 | 0x00,0x00 | 0x00,0x01 | 0x84 | 0x0A |

Answer frame (e.g. reading wind speed2.3 m/s )

| Addre ss code | Funct ional code | Number of valid bytes | Wind speed | Low Checkin g code | High Checking code |
| --- | --- | --- | --- | --- | --- |
| 0x01 | 0x03 | 0x02 | 0x00 0x17 | 0xB8 | 0x41 |

Wind speed:

0017H( hexadecimal)=23=> wind speed =2.3 m/s

  - Notes:

Please check that the package is in good condition and check that the transmitter model and specifications are in accordance with the products you choose. If you have any questions, please contact us as soon as possible.

Please confirm before use: whether the output voltage of the power supply is correct; the positive and negative connection mode of the power supply with the product; and read the product specification or consult our company in detail. Any wiring error  will  cause  irreversible damage to the transmitter.

  - Quality Assurance and After-sale

Quality assurance terms follow Weihai Jingxun unblocked Electronic Technology Co., Ltd. Sensor after-sale terms, for sensor mainframe circuit part of the warranty for two years, gas sensor quality assurance for one year, accessories (shell / plug / cable, etc.) warranty for three months.